# Project Architecture Overview (Google AI Studio)

This design uses a logic level shifter to create two distinct voltage zones:

- 3.3V Control Zone: The ESP32-S3 microcontroller.

- 5V Actuator Zone: All four-digit modules, including their PCF8575 GPIO expanders, stepper drivers, sensors, and LEDs.

This approach eliminates the need for individual voltage dividers on each module, resulting in a cleaner and more reliable build.

### Required Components Checklist

Control:

- ESP32-S3 Super Mini (x1)

- Bi-directional Logic Level Shifter (1x, 2-channel minimum)

Digit Modules (x4 of each):

- PCF8575 I2C GPIO Expander

- ULN2003 Stepper Driver Board

- 28BYJ-48 Stepper Motor

- A3144 Hall Effect Sensor

- Red LED

Passive Components:

- Power Supply: 5V DC, rated for 2 Amps or more.

- 330Ω Resistors (x4): For the homing LEDs.

- 4.7kΩ Resistors (x4): For I2C pull-ups (2 for the 3.3V side, 2 for the 5V side). Note: Some shifter boards have built-in resistors; check your board’s documentation.

- 0.1µF (100nF) Ceramic Capacitors (Optional but Recommended): Place one next to each IC (ESP32 and each PCF8575) for power stability.

### Wiring Instructions

#### Step 1: Power Distribution

- Connect your 5V/2A power supply to create your main 5V Rail and GND Rail.

- Power the ESP32-S3:

  - Connect the 5V Rail to the VIN (or 5V) pin.

  - Connect the GND Rail to the GND pin.

- The 3V3 pin on the ESP32 is now your 3.3V Source.

#### Step 2: Logic Level Shifter & I2C Bus

This shifter bridges the 3.3V ESP32 and the 5V modules.

- Power the Shifter:

  - HV (High Voltage) -> 5V Rail

  - LV (Low Voltage) -> ESP32 3V3 pin

  - GND -> GND Rail

- Connect the ESP32 (Low Voltage Side):

  - ESP32 SDA pin -> Shifter LV1

  - ESP32 SCL pin -> Shifter LV2

  - Add a 4.7kΩ pull-up resistor from LV1 to 3.3V.

  - Add a 4.7kΩ pull-up resistor from LV2 to 3.3V.

- Connect the Modules (High Voltage Side):

  - Shifter HV1 -> SDA pin of all four PCF8575 modules.

  - Shifter HV2 -> SCL pin of all four PCF8575 modules.

  - Add a 4.7kΩ pull-up resistor from HV1 to the 5V Rail.

  - Add a 4.7kΩ pull-up resistor from HV2 to the 5V Rail.

#### Step 3: Set Unique I2C Addresses

This is critical. Each PCF8575 must have a unique address. Set them by connecting the address pins (A0, A1, A2) to either GND (for 0) or 5V (for 1).

| Module | A2 | A1 | A0 | I2C Address |
| --- | --- | --- | --- | --- |
| Digit 1 | GND | GND | GND | 0x20 |
| Digit 2 | GND | GND | 5V | 0x21 |
| Digit 3 | GND | 5V | GND | 0x22 |
| Digit 4 | 5V | GND | GND | 0x23 |

You can use other combinations, just ensure each one is unique.

#### Step 4: Wiring a Single Digit Module (Repeat 4 times)

All components in the module run on 5V.

- Power:

  - PCF8575 VCC -> 5V Rail

  - ULN2003 + -> 5V Rail

  - A3144 VCC -> 5V Rail

  - Connect all GND pins on the module to the common GND Rail.

- Connections:

  - Stepper Motor:

    - Connect the 28BYJ-48 plug into the ULN2003 driver board.

    - PCF8575 P0 -> ULN2003 IN1

    - PCF8575 P1 -> ULN2003 IN2

    - PCF8575 P2 -> ULN2003 IN3

    - PCF8575 P3 -> ULN2003 IN4

  - Homing Sensor (A3144):

    - A3144 Output -> PCF8575 input pin (e.g., P5). No other components are needed.

  - Homing LED:

    - PCF8575 output pin (e.g., P4) -> 330Ω Resistor -> LED Anode (+)

    - LED Cathode (-) -> GND Rail

    - 5V → 330Ω resistor → LED anode; cathode to PCF8575 output (e.g., P05). Active-low.

### Final Sanity Check

Before powering on, review these critical points:

- Power Supply: Ensure your 5V power supply can provide at least 2 Amps. This is the most common cause of instability when motors are running.

- No 5V to ESP32: Confirm that no 5V signals are connected directly to any ESP32 I/O pin. The logic level shifter is the only component bridging the two voltage levels.

- Unique I2C Addresses: Double-check the A0/A1/A2 wiring on each of the four PCF8575 modules. If any two are the same, the I2C bus will fail.

- Grounds are Common: All components (ESP32, shifter, and all four modules) must share a common ground connection.

# Extra (from ChatGPT)

Specific resistors & capacitors to use (concrete)

- I²C pull-ups: 10 kΩ to 3.3 V on ESP side; 4.7 kΩ to 5 V on module side. If bus is long or many devices, use 4.7 kΩ (or 2.2 kΩ for 400 kHz).

- PCF8575 INT pull-up: 10 kΩ to 3.3 V (ESP side).

- Hall sensor (A3144) output pull-up: if you connect A3144 OUT to a PCF8575 input, pull-up to 5 V (10 kΩ) on module (optional if you trust PCF8575 input behavior). If you connect A3144 OUT to ESP32 directly, use 10 kΩ to 3.3 V and confirm open-collector nature.

- LED resistor (5 V drive): 1 kΩ (≈3 mA) or 470 Ω (≈6–7 mA) depending on desired brightness.

- Decoupling: 100 nF ceramic at every IC power pin. Motor bulk: 470–1000 µF electrolytic on the 5 V rail near the motor feed.

- Small 0.1 µF across the A3144 VCC–GND on the module to stabilize sensor supply.

# Checking Level-shifter for built-in pull-ups (Gemini)

Yes, you can absolutely be sure by using a multimeter. It's the perfect tool for the job.

You can also often tell just by looking at the board.

## Using a Multimeter multimeter

Here is the step-by-step process. This must be done with the level shifter completely disconnected and unpowered.

- Prepare Your Multimeter

  - Set your multimeter to measure resistance (Ω).

  - Choose a range like 20kΩ, as most built-in pull-up resistors are between 1kΩ and 10kΩ.

- Test the High Voltage (HV) Side

  - Place one probe on the HV pin.

  - Place the other probe on the SDA_HV pin (sometimes labeled HV2).

  - Check the reading.

  - Now, keep the first probe on the HV pin and move the second probe to the SCL_HV pin (sometimes labeled HV1).

  - Check the reading.

- Test the Low Voltage (LV) Side

  - Place one probe on the LV pin.

  - Place the other probe on the SDA_LV pin (sometimes labeled LV2).

  - Check the reading.

  - Now, keep the first probe on the LV pin and move the second probe to the SCL_LV pin (sometimes labeled LV1).

  - Check the reading.

## Interpreting the Results 💡

- If you see a stable value between 1kΩ and 10kΩ: Congratulations, a pull-up resistor is present on that pin. A reading of "10.0" on the 20kΩ scale means you have a 10kΩ resistor.

- If your meter reads "OL" (Over Limit), "infinite," or just doesn't change from its default state: There is no pull-up resistor connecting those two pins. You will need to add your own.

You should get a consistent reading for both the SDA and SCL pins on a given side (e.g., both pins on the HV side will have 10kΩ pull-ups).

## A Quick Visual Check 👀

You can often spot the resistors just by looking.

Look for small, black surface-mount (SMD) components near the pins. They are usually marked with a numerical code. For a 10kΩ resistor, you will typically see the number "103" (which means 10 followed by 3 zeros, or 10,000). You can follow the copper traces on the board to see if they connect the SDA/SCL pins to the HV/LV pins through these components.